- IEEE Format of 1/5.

- Find the minimal expression for a > b, where a=a0a1 and b=b0b1.

Solution:  [a1’b1] + [a0’b0 and (a1 xor b1)].

- Universal shift register (Refer Morris Mano).

- Data compression is used for information storage and transmission. You are to use encryption along with data compression. What will u do

i) Data compression first and then encrypt the compressed data.

ii) Encrypt the data and then compress the encrypted data.

Justify your answer.

Solution: Compress the data and then encrypt the data.

Reason:

1) Encryption time is proportional to data size. Compressed data size will be less than normal data size.

2) It also hinders cryptanalysis because there's less chance of repeating patterns and moreover, cryptanalysis is always harder when there is less cipher text to chew on.

- Give Binary representation of -125.375 in fixed point 2C notation with 3 bits for fractional part and 8 bits for integer part.

- Given a 4 bit binary number design a circuit that gives square of the number.